APPENDIX-A

ENVIRONMENTAL PARAMETERS

- The following represents the minimum requirements for the stability of proposed submarine pipelines and risers (Field-wise)

  - Mumbai High North Field

    - Submarine Pipeline (Mumbai High North)

|  |  | During Installation | During Operation |
| --- | --- | --- | --- |
| a) | Pipeline Condition | Empty | Full of product |
| b) | Significant Wave Height | 6.30 m | 9.80 m |
| c) | Significant Wave Period | 10.0 Sec. | 12.0 Sec. |
| d) | Wave direction | Perpendicular to Pipeline | Perpendicular to Pipeline |
| e) | Current Velocity at Mud line (Tidal + Wind drift) | 0.40 m/sec | 0.62 m/sec |
| f) | Current Direction | Perpendicular to Pipeline | Perpendicular to Pipeline |
| g) | Force Coefficients: |  |  |
|  | i) Drag Coefficient | 0.75 | 0.75 |
|  | ii) Lift coefficient | 0.75 | 0.75 |
|  | iii)Inertia co-efficient | 3.29 | 3.29 |
| h) | Bottom Friction coefficient |  |  |
|  | i) For thermal analysis | 0.5 | 0.5 |
|  | ii) For lateral Stability | 0.5 | *Based on Soil data as per pre-engg. Survey Report restricted to 0.7 |
| i) | Astronomical Tide | 2.6 m | 3.8 m |
| j) | Storm Surge | 0.3 m | 1.3 m |
| k) | Water depth (Chart datum) |  |  |
|  | - For stress calculation & stability | Min. along route | Min. along route + storm Surge |
|  | - For checking of pipe buckling | Max. along route + max. tide + storm surge + crest | Max. along route + max. tide + storm surge + crest |
| l) | Sea water density (Kg/m3) | 1030 | 1030 |

    - Riser (Mumbai High North)

| A. | Environmental Parameters: |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | During Installation |  | During Operation |
| a) | Max. wave height | 11.58 m |  | 18.0 m |
| b) | Max. wave period | 11.0 sec |  | 14.4 sec. |
| c) | Astronomical tide | 2.6 m |  | 3.8 m |
| d) | Storm tide/total tide | 0.3 m/2.9 m |  | 1.3 m/5.1 m |
| e) | Current velocity |  |  |  |
|  | i) Bottom (mud line) | 0.4 m/sec |  | 0.62 m/sec |
|  | ii) 1/4 depth | 0.7 m/sec |  | 1.1 m/sec |
|  | iii) 1/2 depth | 0.85 m/sec |  | 1.3 m/sec |
|  | iv) 3/4 depth | 1.0 m/sec |  | 1.5 m/sec |
|  | v) Surface | 1.2 m/sec |  | 1.8 m/sec |
| B. | The risers shall be designed for operational condition considering the following marine growth: |  |  |  |
| - | from mud line to 30m below chart datum |  | 50 mm (on radius) |  |
| - | from 30 m below chart datum to EL. (-) 2.0 M CD |  | 100 mm (on radius) |  |
| - | from EL. (-) 2.0 M CD to EL. (+) 6.0 M CD |  | 150 mm (on radius) |  |
|  | The marine growth density shall be taken as 1400 kg/m³ |  |  |  |
| C. | The riser shall be provided with splash zone external corrosion allowance in addition to Monel wrap. This allowance shall be as per Table 4.2A enclosed. |  |  |  |
| D. | The splash zone for the riser is defined as zone between elevation (-) 2.0 m and up to the bottom of hanger clamp, or (+) 6.0 m elevation w.r.t CD, whichever is higher. |  |  |  |
| E. | Internal corrosion allowance in wall thickness for steel pipeline and risers shall be as per Table 4.2A enclosed. |  |  |  |
| F. | Other Parameters |  |  |  |
|  | Force Coefficients | During Installation |  | During Operation |
|  | Drag Coefficient, CD | 0.6 |  | 0.6 |
|  | Inertia Coefficient, CM | 2.0 |  | 2.0 |
| G. | Sea Water Temperature |  |  |  |
|  | Minimum |  | 22.8 °C |  |
|  | Maximum |  | 30.0 °C |  |
| H. | Air Temperature |  |  |  |
|  | Minimum |  | 16 °C |  |
|  | Maximum |  | 40 °C |  |

  - Mumbai High South Field

    - Submarine Pipeline (Mumbai High South)

|  |  | During Installation | During Operation |
| --- | --- | --- | --- |
| a) | Pipeline Condition | Empty | Full of product |
| b) | Significant Wave Height | 6.40 m | 10.20 m |
| c) | Significant Wave Period | 11.0 Sec. | 14.4 Sec. |
| d) | Wave direction | Perpendicular to Pipeline | Perpendicular to Pipeline |
| e) | Current Velocity at Mud line (Tidal + Wind drift) | 0.40 m/sec | 0.51 m/sec |
| f) | Current Direction | Perpendicular to Pipeline | Perpendicular to Pipeline |
| g) | Force Coefficients: |  |  |
|  | i) Drag Coefficient | 0.75 | 0.75 |
|  | ii) Lift coefficient | 0.75 | 0.75 |
|  | iii)Inertia co-efficient | 3.29 | 3.29 |
| h) | Bottom Friction coefficient |  |  |
|  | i) For thermal analysis | 0.5 | 0.5 |
|  | ii) For lateral Stability | 0.5 | *Based on Soil data as per pre-Engg. Survey Report restricted to 0.7 |
| i) | Astronomical Tide | 2.6 m | 3.7 m |
| j) | Storm Surge | 0.3 m | 1.2 m |
| k) | Water depth (Chart datum) |  |  |
|  | - For stress calculation & stability | Min. along route | Min. along route + storm Surge |
|  | - For checking of pipe buckling | Max. along route + max. tide + storm surge + crest | Max. along route + max. tide + storm surge + crest |
| l) | Sea water density (Kg/m3) | 1030 | 1030 |

    - Riser (Mumbai High South)

| A. | Environmental Parameters: |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | During Installation |  | During Operation |
| a) | Max. wave height | 11.58 m |  | 18.3 m |
| b) | Max. wave period | 11.0 sec |  | 14.4 sec. |
| c) | Astronomical tide | 2.6 m |  | 3.7 m |
| d) | Storm tide/total tide | 0.3 m/2.9 m |  | 1.2 m/4.9 m |
| e) | Current velocity |  |  |  |
|  | i) Bottom (mud line) | 0.4 m/sec |  | 0.51 m/sec |
|  | ii) 1/4 depth | 0.7 m/sec |  | 1.0 m/sec |
|  | iii) 1/2 depth | 0.85 m/sec |  | 1.2 m/sec |
|  | iv) 3/4 depth | 1.0 m/sec |  | 1.4 m/sec |
|  | v) Surface | 1.2 m/sec |  | 1.65 m/sec |
| B. | The risers shall be designed for operational condition considering the following marine growth: |  |  |  |
| - | from mud line to 30 m below chart datum |  | 50 mm (on radius) |  |
| - | from 30 m below chart datum to EL. (-) 2.0 M CD |  | 100 mm (on radius) |  |
| - | from EL. (-) 2.0 M CD to EL. (+) 6.0 M CD |  | 150 mm (on radius) |  |
|  | The marine growth density shall be taken as 1400 kg/m³ |  |  |  |
| C. | The riser shall be provided with splash zone external corrosion allowance in addition to Monel wrap. This allowance shall be as per Table 4.2A enclosed. |  |  |  |
| D. | The splash zone for the riser is defined as zone between elevation (-) 2.0 m and up to the bottom of hanger clamp, or (+) 6.0 m elevation w.r.t CD, whichever is higher. |  |  |  |
| E. | Internal corrosion allowance in wall thickness for steel pipeline and risers shall be as per Table 4.2A enclosed. |  |  |  |
| F. | Other Parameters |  |  |  |
|  | Force Coefficients | During Installation |  | During Operation |
|  | Drag Coefficient, CD | 0.6 |  | 0.6 |
|  | Inertia Coefficient, CM | 2.0 |  | 2.0 |
| G. | Sea Water Temperature |  |  |  |
|  | Minimum |  | 22.8 °C |  |
|  | Maximum |  | 30.0 °C |  |
| H. | Air Temperature |  |  |  |
|  | Minimum |  | 16 °C |  |
|  | Maximum |  | 40 °C |  |

  - Neelam Field

    - Submarine Pipeline (Neelam)

|  |  | During Installation | During Operation |
| --- | --- | --- | --- |
| a) | Pipeline Condition | Empty | Full of product |
| b) | Significant Wave Height | 7.2 m | 9.7 m |
| c) | Significant Wave Period | 10.16 Sec. | 10.69 Sec. |
| d) | Wave Direction | Perpendicular to Pipeline | Perpendicular to Pipeline |
| e) | Current Velocity at Mud Line (Tidal + Wind Drift) | 0.44 m/sec | 0.44 m/sec |
| f) | Current Direction | Perpendicular to Pipeline | Perpendicular to Pipeline |
| g) | Force Coefficients: |  |  |
|  | i) Drag Coefficient | 0.75 | 0.75 |
|  | ii) Lift Coefficient | 0.75 | 0.75 |
|  | iii)Inertia Coefficient | 3.29 | 3.29 |
| h) | Bottom Friction Coefficient |  |  |
|  | i) For Thermal Analysis | i) For Thermal Analysis | i) For Thermal Analysis |
|  | ii) For Lateral Stability | ii) For Lateral Stability | ii) For Lateral Stability |
|  | *In case, the value of Bottom Friction Coefficient as per pre-Engg. Survey exceeds 0.7; same shall be restricted to 0.7 for On-Bottom Stability Design. |  |  |
| i) | Astronomical Tide | 3.66 m | 4.6 m |
| j) | Storm Surge | 0.61 m | 1.23 m |
| k) | Water depth (Chart datum) |  |  |
|  | - For stress calculation & stability | Min. along route | Min. along route + storm Surge |
|  | - For checking of pipe buckling | Max. along route + max. tide + storm surge + crest | Max. along route + max. tide + storm surge + crest |
| l) | Sea water density (Kg/m3) | 1030 | 1030 |

    - Riser (Neelam)

| A. | Environmental Parameters: |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | During Installation |  | During Operation |
| a) | Max. wave height | 11.58 m |  | 18.0 m |
| b) | Max. wave period | 11.0 sec |  | 14.4 sec |
| c) | Astronomical tide | 3.66 m |  | 4.6 m |
| d) | Storm tide/total tide | 0.61 m / 4.27 m |  | 1.23 m / 5.83 m |
| e) | Current velocity |  |  |  |
|  | i) Bottom (mud line) | 0.44 m/sec |  | 0.44 m/sec |
|  | ii) 1/4 depth | 0.77 m/sec |  | 0.90 m/sec |
|  | iii) 1/2 depth | 0.92 m/sec |  | 1.12 m/sec |
|  | iv) 3/4 depth | 1.1 m/sec |  | 1.31 m/sec |
|  | v) Surface | 1.25 m/sec |  | 1.55 m/sec |
| B. | The risers shall be designed for operational condition considering the following marine growth: |  |  |  |
| - | from mud line to 30 m below chart datum |  | from mud line to 30 m below chart datum |  |
| - | from 30 m below chart datum to EL. (-) 2.0 M CD |  | from 30 m below chart datum to EL. (-) 2.0 M CD |  |
| - | from EL. (-) 2.0 M CD to EL. (+) 6.0 M CD |  | from EL. (-) 2.0 M CD to EL. (+) 6.0 M CD |  |
|  | The marine growth density shall be taken as 1400 kg/m³ |  |  |  |
| C. | The riser shall be provided with splash zone external corrosion allowance in addition to Monel wrap. This allowance shall be as per Table 4.2A enclosed. |  |  |  |
| D. | The splash zone for the riser is defined as zone between elevation (-) 2.0 m and up to the bottom of hanger clamp, or (+) 6.0 m elevation w.r.t CD, whichever is higher. |  |  |  |
| E. | Internal corrosion allowance in wall thickness for steel pipeline and risers shall be as per Table 4.2A enclosed. |  |  |  |
| F. | Other Parameters |  |  |  |
|  | Force Coefficients | During Installation |  | During Operation |
|  | Drag Coefficient, CD | 0.6 |  | 0.6 |
|  | Inertia Coefficient, CM | 2.0 |  | 2.0 |
| G. | Sea Water Temperature |  |  |  |
|  | Minimum |  | 22.8 °C |  |
|  | Maximum |  | 30.0 °C |  |
| H. | Air Temperature |  |  |  |
|  | Minimum |  | 16 °C |  |
|  | Maximum |  | 40 °C |  |

  - Heera and Ratna Fields

    - Submarine Pipeline (Heera and Ratna)

|  |  | During Installation | During Operation |
| --- | --- | --- | --- |
| a) | Pipeline Condition | Empty | Full of product |
| b) | Significant Wave Height | 6.61 m | 9.14 m |
| c) | Significant Wave Period | 10.3 Sec. | 12.3 Sec. |
| d) | Wave direction | Perpendicular to Pipeline | Perpendicular to Pipeline |
| e) | Current Velocity at Mud line (Tidal + Wind drift) | 0.45 m/sec | 0.45 m/sec |
| f) | Current Direction | Perpendicular to Pipeline | Perpendicular to Pipeline |
| g) | Force Coefficients: |  |  |
|  | i) Drag Coefficient | 0.75 | 0.75 |
|  | ii) Lift coefficient | 0.75 | 0.75 |
|  | iii)Inertia co-efficient | 3.29 | 3.29 |
| h) | Bottom Friction coefficient |  |  |
|  | i) For thermal analysis | 0.5 | 0.5 |
|  | ii) For lateral Stability | 0.5 | *Based on Soil data as per pre-engg. Survey Report restricted to 0.7 |
| i) | Astronomical Tide | 3.66 m | 4.0 m |
| j) | Storm Surge | 0.61 m | 1.3 m |
| k) | Water depth (Chart datum) |  |  |
|  | - For stress calculation & stability | Min. along route | Min. along route + storm Surge |
|  | - For checking of pipe buckling | Max. along route + max. tide + storm surge + crest | Max. along route + max. tide + storm surge + crest |
| l) | Sea water density (Kg/m3) | 1030 | 1030 |

    - Riser (Heera and Ratna)

| A. | Environmental Parameters: |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | During Installation |  | During Operation |
| a) | Max. wave height | 11.58 m |  | 17.7 m |
| b) | Max. wave period | 11.0 sec |  | 14.3 sec. |
| c) | Astronomical tide | 3.66 m |  | 4.0 m |
| d) | Storm tide/total tide | 0.61 m/4.27 m |  | 1.3 m/5.3 m |
| e) | Current velocity |  |  |  |
|  | i) Bottom (mud line) | 0.45 m/sec |  | 0.45 m/sec |
|  | ii) 1/4 depth | 0.878 m/sec |  | 0.94 m/sec |
|  | iii) 1/2 depth | 1.049 m/sec |  | 1.16 m/sec |
|  | iv) 3/4 depth | 1.22 m/sec |  | 1.37 m/sec |
|  | v) Surface | 1.387 m/sec |  | 1.60 m/sec |
| B. | The risers shall be designed for operational condition considering the following marine growth: |  |  |  |
| - | from mud line to 30 m below chart datum |  | 50 mm (on radius) |  |
| - | from 30 m below chart datum to EL. (-) 2.0 M CD |  | 100 mm (on radius) |  |
| - | from EL. (-) 2.0 M CD to EL. (+) 6.0 M CD |  | 150 mm (on radius) |  |
|  | The marine growth density shall be taken as 1400 kg/m³ |  |  |  |
| C. | The riser shall be provided with splash zone external corrosion allowance in addition to Monel wrap. This allowance shall be as per Table 4.2A enclosed. |  |  |  |
| D. | The splash zone for the riser is defined as zone between elevation (-) 2.0 m and up to the bottom of hanger clamp, or (+) 6.0 m elevation w.r.t CD, whichever is higher. |  |  |  |
| E. | Internal corrosion allowance in wall thickness for steel pipeline and risers shall be as per Table 4.2A enclosed. |  |  |  |
| F. | Other Parameters |  |  |  |
|  | Force Coefficients | During Installation |  | During Operation |
|  | Drag Coefficient, CD | 0.6 |  | 0.6 |
|  | Inertia Coefficient, CM | 2.0 |  | 2.0 |
| G. | Sea Water Temperature |  |  |  |
|  | Minimum |  | 22.8 °C |  |
|  | Maximum |  | 30.0 °C |  |
| H. | Air Temperature |  |  |  |
|  | Minimum |  | 16 °C |  |
|  | Maximum |  | 40 °C |  |

  - Bassein and Panna Fields

    - Submarine Pipeline (Bassein and Panna)

|  |  | During Installation | During Operation |
| --- | --- | --- | --- |
| a) | Pipeline Condition | Empty | Full of product |
| b) | Significant Wave Height | 5.60 m | 8.70 m |
| c) | Significant Wave Period | 10.0 Sec. | 13.3 Sec. |
| d) | Wave direction | Perpendicular to Pipeline | Perpendicular to Pipeline |
| e) | Current Velocity at Mud line (Tidal + Wind drift) | 0.52 m/sec | 0.67 m/sec |
| f) | Current Direction | Perpendicular to Pipeline | Perpendicular to Pipeline |
| g) | Force Coefficients: |  |  |
|  | i) Drag Coefficient | 0.75 | 0.75 |
|  | ii) Lift coefficient | 0.75 | 0.75 |
|  | iii) Inertia co-efficient | 3.29 | 3.29 |
| h) | Bottom Friction coefficient |  |  |
|  | i) For thermal analysis | 0.5 | 0.5 |
|  | ii) For lateral Stability | 0.5 | *Based on Soil data as per pre-engg. Survey Report restricted to 0.7 |
| i) | Astronomical Tide | 3.1 m | 4.4 m |
| j) | Storm Surge | 0.61 m | 1.2 m |
| k) | Water depth (Chart datum) |  |  |
|  | - For stress calculation & stability | Min. along route | Min. along route + storm Surge |
|  | - For checking of pipe buckling | Max. along route + max. tide + storm surge + crest | Max. along route + max. tide + storm surge + crest |
| l) | Sea water density (Kg/m3) | 1030 | 1030 |

    - Riser (Bassein and Panna)

| A. | Environmental Parameters: |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | During Installation |  | During Operation |
| a) | Max. wave height | 10.4 m |  | 18.0 m |
| b) | Max. wave period | 11.9 sec |  | 14.4 sec. |
| c) | Astronomical tide | 3.1 m |  | 4.4 m |
| d) | Storm tide/total tide | 0.6 m/3.7 m |  | 1.2 m/5.6 m |
| e) | Current velocity |  |  |  |
|  | i) Bottom (mud line) | 0.52 m/sec |  | 0.67 m/sec |
|  | ii) 1/4 depth | 0.753 m/sec |  | 1.05 m/sec |
|  | iii) 1/2 depth | 0.832 m/sec |  | 1.26 m/sec |
|  | iv) 3/4 depth | 0.881 m/sec |  | 1.48 m/sec |
|  | v) Surface | 0.918 m/sec |  | 1.73 m/sec |
| B. | The risers shall be designed for operational condition considering the following marine growth: |  |  |  |
| - | from mud line to 30 m below chart datum |  | 50 mm (on radius) |  |
| - | from 30 m below chart datum to EL. (-) 2.0 M CD |  | 100 mm (on radius) |  |
| - | from EL. (-) 2.0 M CD to EL. (+) 6.0 M CD |  | 150 mm (on radius) |  |
|  | The marine growth density shall be taken as 1400 kg/m³ |  |  |  |
| C. | The riser shall be provided with splash zone external corrosion allowance in addition to Monel wrap. This allowance shall be as per Table 4.2A enclosed. |  |  |  |
| D. | The splash zone for the riser is defined as zone between elevation (-) 2.0 m and up to the bottom of hanger clamp, or (+) 6.0 m elevation w.r.t CD, whichever is higher. |  |  |  |
| E. | Internal corrosion allowance in wall thickness for steel pipeline and risers shall be as per Table 4.2A enclosed. |  |  |  |
| F. | Other Parameters |  |  |  |
|  | Force Coefficients | During Installation |  | During Operation |
|  | Drag Coefficient, CD | 0.6 |  | 0.6 |
|  | Inertia Coefficient, CM | 2.0 |  | 2.0 |
| G. | Sea Water Temperature |  |  |  |
|  | Minimum |  | 22.8 °C |  |
|  | Maximum |  | 30.0 °C |  |
| H. | Air Temperature |  |  |  |
|  | Minimum |  | 16 °C |  |
|  | Maximum |  | 40 °C |  |

  - Cluster-7 Marginal Field

    - Submarine Pipeline (Cluster-7)

|  |  | During Installation | During Operation |
| --- | --- | --- | --- |
| a) | Pipeline Condition | Empty | Full of product |
| b) | Significant Wave Height | 6.40 m | 10.20 m |
| c) | Significant Wave Period | 11.0 Sec. | 14.4 Sec. |
| d) | Wave direction | Perpendicular to Pipeline | Perpendicular to Pipeline |
| e) | Current Velocity at Mud line (Tidal + Wind drift) | 0.40 m/sec | 0.51 m/sec |
| f) | Current Direction | Perpendicular to Pipeline | Perpendicular to Pipeline |
| g) | Force Coefficients: |  |  |
|  | i) Drag Coefficient | 0.75 | 0.75 |
|  | ii) Lift coefficient | 0.75 | 0.75 |
|  | iii)Inertia co-efficient | 3.29 | 3.29 |
| h) | Bottom Friction coefficient |  |  |
|  | i) For thermal analysis | 0.5 | 0.5 |
|  | ii) For lateral Stability | 0.5 | *Based on Soil data as per pre-Engg. Survey Report restricted to 0.7 |
| i) | Astronomical Tide | 2.6 m | 3.7 m |
| j) | Storm Surge | 0.3 m | 1.2 m |
| k) | Water depth (Chart datum) |  |  |
|  | - For stress calculation & stability | Min. along route | Min. along route + storm Surge |
|  | - For checking of pipe buckling | Max. along route + max. tide + storm surge + crest | Max. along route + max. tide + storm surge + crest |
| l) | Sea water density (Kg/m3) | 1030 | 1030 |

    - Riser (Cluster-7)

| A. | Environmental Parameters: |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | During Installation |  | During Operation |
| a) | Max. wave height | 11.60 m |  | 18.0 m |
| b) | Max. wave period | 11.0 sec |  | 14.4 sec. |
| c) | Astronomical tide | 2.6 m |  | 3.8 m |
| d) | Storm tide/total tide | 0.3 m/2.9 m |  | 1.3 m/5.1 m |
| e) | Current velocity |  |  |  |
|  | i) Bottom (mud line) | 0.4 m/sec |  | 0.62 m/sec |
|  | ii) 3/4 depth | 0.7 m/sec |  | 1.1 m/sec |
|  | iii) 1/2 depth | 0.85 m/sec |  | 1.3 m/sec |
|  | iv) 1/4 depth | 1.0 m/sec |  | 1.5 m/sec |
|  | v) Surface | 1.2 m/sec |  | 1.8 m/sec |
| B1 | Risers shall be designed for operational condition considering the following marine growth: |  |  |  |
| - | from mud line to 30 m below chart datum |  | 50 mm (on radius) |  |
| - | from 30 m below chart datum to EL. (+) 3.0 M CD |  | 100 mm (on radius) |  |
| B2 | The risers on new platforms shall be designed for operational condition considering the following marine growth: |  |  |  |
| - | from mud line to 30 m below chart datum |  | 50 mm (on radius) |  |
| - | from 30 m below chart datum to EL. (+) 3.0 M CD |  | 225 mm (on radius) |  |
|  | The marine growth density shall be taken as 1030 kg/m³ |  |  |  |
| C. | The riser shall be provided with splash zone external corrosion allowance in addition to Monel wrap. This allowance shall be as per Table 4.2A enclosed. |  |  |  |
| D. | The splash zone for the riser is defined as zone between elevation (-) 2.0 m and up to the bottom of hanger clamp, or (+) 6.0 m elevation w.r.t CD, whichever is higher. |  |  |  |
| E. | Internal corrosion allowance in wall thickness for steel pipeline and risers shall be as per Table 4.2A enclosed. |  |  |  |
| F. | Other Parameters |  |  |  |
|  | Force Coefficients | During Installation |  | During Operation |
|  | Drag Coefficient, CD | 0.6 |  | 0.6 |
|  | Inertia Coefficient, CM | 2.0 |  | 2.0 |
| G. | Sea Water Temperature |  |  |  |
|  | Minimum |  | 22.8 °C |  |
|  | Maximum |  | 30.0 °C |  |
| H. | Air Temperature |  |  |  |
|  | Minimum |  | 16 °C |  |
|  | Maximum |  | 40 °C |  |

  - Daman Field

    - Submarine Pipeline (Daman)

|  |  | During Installation | During Operation |
| --- | --- | --- | --- |
| a) | Pipeline Condition | Empty | Full of product |
| b) | Significant Wave Height | 5.243 m | 9.418 m |
| c) | Significant Wave Period | 9.2 sec | 13 sec |
| d) | Wave direction | Perpendicular to Pipeline | Perpendicular to Pipeline |
| e) | Current Velocity at Mud line (Tidal + Wind drift) | 0.579 m/sec | 0.762 m/sec |
| f) | Current Direction | Perpendicular to Pipeline | Perpendicular to Pipeline |
| g) | Force Coefficients: |  |  |
|  | i) Drag Coefficient | 0.75 | 0.75 |
|  | ii) Lift coefficient | 0.75 | 0.75 |
|  | iii) Inertia co-efficient | 3.29 | 3.29 |
| h) | Bottom Friction coefficient |  |  |
|  | i) For thermal analysis | 0.5 | 0.5 |
|  | ii) For lateral Stability | 0.5 | Based on Soil data as per pre-Engg. Survey Report restricted to 0.7 maximum. |
| i) | Astronomical Tide | 4.267 m | 4.267 m |
| j) | Storm Surge | 0.884 m | 1.494 m |
| k) | Water depth (Chart datum) |  |  |
|  | - i) For stress calculation & stability | Min. along route | Min. along route + storm surge |
|  | - ii) For checking of pipe buckling | Max. along route + max. tide + storm surge + crest | Max. along route + max. tide + storm surge + crest |
| l) | Sea water density (Kg/m3) | 1030 | 1030 |

    - Riser (Daman)

| A. | Environmental Parameters: |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | During Installation |  | During Operation |
| a) | Max. wave height | 9.754 m |  | 17.496 m |
| b) | Max. wave period | 10.2 sec |  | 14.4 sec |
| c) | Astronomical tide | 4.267 m |  | 4.267 m |
| d) | Storm tide/total tide | 0.884 m/5.151 m |  | 1.494 m/5.761 m |
| e) | Current velocity |  |  |  |
|  | i) Bottom (mud line) | 0.579 m/sec |  | 0.762 m/sec |
|  | ii) 3/4 depth | 0.914 m/sec |  | 1.173 m/sec |
|  | iii) 1/2 depth | 1.067 m/sec |  | 1.402 m/sec |
|  | iv) 1/4 depth | 1.250 m/sec |  | 1.631 m/sec |
|  | v) Surface | 1.402 m/sec |  | 1.859 m/sec |
| B. | Risers shall be designed for operational condition considering the following marine growth: |  |  |  |
| - | from mud line to 30m below chart datum |  | 50 mm (on radius) |  |
| - | from 30 m below chart datum to EL. (-) 2.0 M CD |  | 100 mm (on radius) |  |
| - | from - 2 m below chart datum to EL. (+) 6.0 M CD |  | 150 mm (on radius) |  |
|  | The marine growth density shall be taken as 1400 kg/m³ |  |  |  |
| C. | The riser shall be provided with splash zone external corrosion allowance in addition to Monel wrap. This allowance shall be as per Table 4.2A enclosed. |  |  |  |
| D. | The splash zone for the riser is defined as zone between elevation (-) 2.0 m and up to the bottom of hanger clamp, or (+) 6.0 m elevation w.r.t CD, whichever is higher. |  |  |  |
| E. | Internal corrosion allowance in wall thickness for steel pipeline and risers shall be as per Table 4.2A enclosed. |  |  |  |
| F. | Other Parameters |  |  |  |
|  | Force Coefficients | During Installation |  | During Operation |
|  | Drag Coefficient, CD | 0.6 |  | 0.6 |
|  | Inertia Coefficient, CM | 2.0 |  | 2.0 |
| G. | Sea Water Temperature |  |  |  |
|  | Minimum |  | 22.8 °C |  |
|  | Maximum |  | 30.0 °C |  |
| H. | Air Temperature |  |  |  |
|  | Minimum |  | 16 °C |  |
|  | Maximum |  | 40 °C |  |

  - NBP Field

    - Submarine Pipeline (NBP)

|  |  | During Installation | During Operation |
| --- | --- | --- | --- |
| a) | Pipeline Condition | Empty | Full of product |
| b) | Significant Wave Height | 6.40 m | 10.20 m |
| c) | Significant Wave Period | 11.0 Sec. | 14.4 Sec. |
| d) | Wave direction | Perpendicular to Pipeline | Perpendicular to Pipeline |
| e) | Current Velocity at Mud line (Tidal + Wind drift) | 0.40 m/sec | 0.51 m/sec |
| f) | Current Direction | Perpendicular to Pipeline | Perpendicular to Pipeline |
| g) | Force Coefficients: |  |  |
|  | i) Drag Coefficient | 0.75 | 0.75 |
|  | ii) Lift coefficient | 0.75 | 0.75 |
|  | iii)Inertia co-efficient | 3.29 | 3.29 |
| h) | Bottom Friction coefficient |  |  |
|  | i) For thermal analysis | 0.5 | 0.5 |
|  | ii) For lateral Stability | 0.5 | *Based on Soil data as per pre-Engg. Survey Report restricted to 0.7 |
| i) | Astronomical Tide | 3.66 m | 3.66 m |
| j) | Storm Surge | 0.61 m | 1.16 m |
| k) | Water depth (Chart datum) |  |  |
|  | - For stress calculation & stability | Min. along route | Min. along route + storm Surge |
|  | - For checking of pipe buckling | Max. along route + max. tide + storm surge + crest | Max. along route + max. tide + storm surge + crest |
| l) | Sea water density (Kg/m3) | 1030 | 1030 |

    - Riser (NBP)

| A. | Environmental Parameters: |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | During Installation |  | During Operation |
| a) | Max. wave height | 11.60 m |  | 18.0 m |
| b) | Max. wave period | 11.0 sec |  | 14.4 sec. |
| c) | Astronomical tide | 3.66 m |  | 3.66 m |
| d) | Storm tide/total tide | 0.61 m/4.27 m |  | 1.16 m/4.82 m |
| e) | Current velocity |  |  |  |
|  | i) Bottom (mud line) | 0.45 m/sec |  | 0.51 m/sec |
|  | ii) 3/4 depth | 0.77 m/sec |  | 0.97 m/sec |
|  | iii) 1/2 depth | 0.92 m/sec |  | 1.19 m/sec |
|  | iv) 1/4 depth | 1.10 m/sec |  | 1.40 m/sec |
|  | v) Surface | 1.25 m/sec |  | 1.64 m/sec |
| B. | Risers shall be designed for operational condition considering the following marine growth: |  |  |  |
| - | from mud line to 30 m below chart datum |  | 50 mm (on radius) |  |
| - | from 30 m below chart datum to EL. (-) 2.0 M CD |  | 100 mm (on radius) |  |
| - | from EL. (-) 2.0 M CD to EL. (+) 6.0 M CD |  | 150 mm (on radius) |  |
|  | The marine growth density shall be taken as 1400 kg/m³ |  |  |  |
| C. | The riser shall be provided with splash zone external corrosion allowance in addition to Monel wrap. This allowance shall be as per Table 4.2A enclosed. |  |  |  |
| D. | The splash zone for the riser is defined as zone between elevation (-) 2.0 m and up to the bottom of hanger clamp, or (+) 6.0 m elevation w.r.t CD, whichever is higher. |  |  |  |
| E. | Internal corrosion allowance in wall thickness for steel pipeline and risers shall be as per Table 4.2A enclosed. |  |  |  |
| F. | Other Parameters |  |  |  |
|  | Force Coefficients | During Installation |  | During Operation |
|  | Drag Coefficient, CD | 0.6 |  | 0.6 |
|  | Inertia Coefficient, CM | 2.0 |  | 2.0 |
| G. | Sea Water Temperature |  |  |  |
|  | Minimum |  | 22.8 °C |  |
|  | Maximum |  | 30.0 °C |  |
| H. | Air Temperature |  |  |  |
|  | Minimum |  | 16 °C |  |
|  | Maximum |  | 40 °C |  |

APPENDIX-B

JACKET DEFLECTIONS

- Following jacket movements for well and process platforms from mud line to hanger clamps are to be considered:

  - Mumbai High North Field

100-year storm			:	200 mm

1 year storm 			: 	100 mm

  - Mumbai High South Field

100-year storm			:	150 mm

1 year storm 			: 	100 mm

  - Neelam Field

100-year storm

North/South	Mudline	:	125 mm

Spider Deck	:	217 mm

East/West	Mudline	:	61 mm

Spider Deck	:	125 mm

1 year storm

North/South	Mudline	:	125 mm

Spider Deck 	:	217 mm

East/West	Mudline 	:	61 mm

Spider Deck	:	125 mm

  - Heera Field

100-year storm			:	400 mm

1 year storm			:	300 mm

  - Bassein & Panna Fields

100-year storm			:	200 mm

1 year storm			:	200 mm

APPENDIX-C

WAVE EXCEEDANCE TABLE

- Number of Waves Exceeding Specified Height in 1 Year for Offshore Mumbai Area

| WAVE HEIGHT (Ft.) | NUMBER OF WAVE EXCEEDANCES |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | S DIR | SW DIR | W DIR | NW DIR | ALL DIR |
| 0 | 1276045 | 770535 | 1015713 | 1220511 | 4282804 |
| 5 | 61704 | 219347 | 220985 | 69788 | 571824 |
| 10 | 3132 | 37929 | 31902 | 3764 | 76727 |
| 15 | 167 | 5878 | 4073 | 177 | 10295 |
| 20 | 11 | 869 | 493 | 8 | 1381 |
| 25 | 0 | 126 | 59 | 0 | 185 |
| 30 | - | 18 | 7 | - | 25 |
| 35 | 2 | 1 | - | - | 3 |
| 40 | - | 0 | - | 0 | - |

  - Number of Waves Exceeding Specified Height in 1 Year for Daman Area

| WAVE HEIGHT (m) | NUMBER OF WAVE EXCEEDANCES |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | S DIR | SW DIR | W DIR | NW DIR | ALL DIR |
| 0 | 1,77,348 | 10,84,956 | 15,64,841 | 9,96,281 | 38,23,426 |
| 0.61 | 69,102 | 4,22,740 | 6,09,722 | 3,88,189 | 14,89,753 |
| 1.22 | 26,282 | 1,60,783 | 2,31,898 | 1,47,642 | 5,66,605 |
| 1.83 | 9,996 | 61,151 | 88,198 | 56,153 | 2,15,498 |
| 2.44 | 3,802 | 23,258 | 33,545 | 21,357 | 81,962 |
| 3.05 | 1,446 | 8,846 | 12,758 | 8,123 | 31,173 |
| 3.66 | 550 | 3,365 | 4,853 | 3,089 | 11,857 |
| 4.27 | 225 | 1,380 | 1,989 | 1,175 | 4,769 |
| 4.88 | 137 | 836 | 1,206 | 47 | 2,226 |
| 5.49 | 56 | 342 | 492 | - | 890 |
| 6.1 | 21 | 130 | 187 | - | 338 |
| 7.62 | 1 | 16 | 13 | - | 30 |
| 9.14 | - | 2 | 1 | - | 3 |
| 10.67 | - | - | - | - | - |

ANNEXURE-1

SUBMARINE PIPELINE DESIGNATION TABLE

ANNEXURE-2

LIST OF SPECIFICATIONS

| SL.NO | DESCRIPTION | SPEC.NO. |
| --- | --- | --- |
|  | Route Survey | 2011 Rev.2 |
|  | Corrosion Protection Coating (3LPP) | 2012A Rev.0 |
|  | Concrete Weight Coating along with addendum to specification | 2013 Rev.1 |
|  | Field Joint Coating along with addendum to Specification | 2014 Rev.0 |
|  | Installation of submarine pipelines & related facilities | 2015 Rev.1 |
|  | Long Radius Bends | 2018 Rev.0 |
|  | C.S. Seamless Line pipe for Submarine Pipeline along with Addendum | 2020A Rev.5 |
|  | C.S. Seamless Line pipe for Submarine Pipeline (sour service) along with Addendum and Addendum-2 to specification | 2020B Rev.5 |
|  | Longitudinal Seam Submerged Arc welded Submarine Pipelines (Sour Service) along with the addendum and Addendum-2 | 2020D Rev.2 |
|  | Unbonded Flexible Pipe For Submarine Application with Addendum | 2020E Rev 4 |
|  | Hydrostatic Testing of Submarine Pipeline | 2022 Rev. 0 |
|  | Fittings and Flanges for Submarine Pipeline | 2024A Rev.0 |
|  | Fittings and Flanges for Submarine Pipeline (Sour Service) | 2024B Rev.0 |
|  | Subsea Ball Valve | 2025B Rev 2 |
|  | Data Sheet – Subsea Ball Valves | 2025B-DS-BV-PRP-VII-RTR Rev 0 |
|  | Subsea Flow TEE (Sour Service) | 2028B Rev 0 |
|  | Data Sheet – Subsea Flow Tee (Sour Service) | 2028B-DS-TEE-PRP-VII-RTR Rev 0 |